Documentation and Release notes for the bdm-gdb-patches

G.Magin 20.04.95

Contents:

A. Concept

B. Features

C. Installing

D. Caveats, Watch-Outs, Gotchas

These are the patches for gdb-4.13 to enable remote debugging for a 683xx

target with BDM under Linux.

BDM (Background Debug Mode) is a special debugging mode for (CPU32{+})

based microcontroller. These controllers are equipped with some pins

especially for debugging purpose. With BDM it is possible to debug an

embedded controller without using any processor resources like serial

interfaces or memory space or special hardware.

Motorola published an hardware interface from a PC's parallel port to the

serial lines of BDM. It consists of just two 74HC chips. With this

interface and the software patches supplied with this, you can perform

source level debugging on an embedded target.

We have used this software setup successfully with a Highscreen Notebook

486DX-33 and Slackware 2.1, kernel versions from 1.1.72 to 1.2.4, and a

Motorola BCC target board. The target code is generated with a GNU

toolchain, i.e. gcc-2.6.3 and binutils-2.5.1, after applying the cross-patch

available from ftp.cygnus.com:/pub/embedded. The C-Libarary for the target

is newlib-1.6 with modified target stubs for the bcc. Newlib-1.6 is also

available from ftp.cygnus.com:/pub/embedded. Alternatively they are

available at ftp.fta-berlin.de:/pub/crossgcc. GNU-sources are available

at most ftp servers near you.

Send suggestions, questions, bugreports etc. to

magin@lpr.e-technik.tu-muenchen.de

--------------------------------------------------------------------------------

A. Concept:

===========

Debugging via BDM is accomplished by two modules:

1. loadable device driver under Linux

2. target module within gdb

A loadable device driver has been written for Linux and the public domain

bdm-interface. This interface hardware has been published by Motorola and

is currently supported by the bd32 debugger for DOS. I have included the

schematic of the interface in both Orcad-format and a format for a Epson

FX-80 compatible printer. The description comes from the file bd32-122.zip

which contains the bd32 debugger. It can be obtained from the Motorola

mailbox or via anonymous ftp from:

bode.ee.ualberta.ca:/pub/motorola/mcu332/bd32-122.zip

freeware.aus.sps.mot.com:/pub/mcu332/bd32-122.zip

To learn more about interfacing to the bdm interface, get the application note

AN1230/D from Motorola and the file BDM-V090.ZIP which contains the companion

source code for DOS. You get BDM-V090.ZIP at the same location as bd32-122.zip.

The target module within gdb and the bdm driver communicate via the

standard open/read/write/ioctl-interface.  After having applied the patches,

see gdb-4.13/gdb/bdm.h for the supported ioctls.

The data to be read and written by the driver are the 16-byte words needed and

supplied by CPU32.  See CPU32 manual, chapter 7 (Development Support) for

what commands are possible.

--------------------------------------------------------------------------------

B. Features:

============

Enabling the bdm target within gdb is performed with the `target' command.

Use `target bdm /dev/bdmx', where x is the parallel port where the bdm

interface sits.

Download a program:

If you issue a "run" command, you will be asked, if you want a download,

and if yes, the program will be downloaded to the target and started, if

no, the program is just started. You will be informed, what program you

have downloaded before, so you might want to skip the download and save

some seconds.

If you want to watch the progress of download, you might issue the

"set hash" command. Then, for every 512 Bytes of downloaded code, a `.'

gets displayed.

Breakpoints:

Anything you can do with a local debugger concerning breakpoints can be done

with the remote debugger, eg. fixed bp, temporary bp, counting bp, enable

and disable bp, watchpoints, commands, etc.

Interrupt:

With ctrl-c you can interrupt your running program on the target and get to

the gdb prompt.

Singlestepping:

next and step work like in a host debugger. As well as nexti and stepi.

Continue:

Your program will continue after that command.

Variables:

Examine and set variables and memory areas with "print" and "examine"

Registers:

Examine and set processor registers of the user model (i.e. D0-D7,A0-A5,FP,SP)

with standard gdb commands (info registers, print $d0, etc.)

Examine and set other processor registers (supervisor model) with bdm

enhancement commands: All registers accessible via bdm are also

accessible by gdb via `bdmget' and `bdmset'. Registers are vbr, ssp, usp,

rpc, ppc, sfc, dfc, sr, far, atemp. However, usage of atemp, far and rpc/ppc

is not recommended, as the bdm target module uses and modifies it.

Macro Facility:

This is an enhancement for the download feature. The motivation is, I had

to enable a memory expansion on my BCC-Board, before being able to actually

download the code.

To use the macro facility you have to supply a macro file named after your

file you want to download. If you want to download `/usr/proj/remote/rob.coff'

the macro file should be named `/usr/proj/remote/rob.bdmmb'. (bdm stands

for bdm-debugger, mb for Macro-Before-download). If there is no macro file,

or there is some trouble opening it, this is silently ignored, and download

proceeds without macro file.

The syntax of the macro file is:

<cmd-letter> <num1> <num2> <num3>

The nums are either in hex (form 0x), in decimal (form 123) or in

octal (form 0234)

The meaning of the nums depends on the command letter:

w or W means: (write)

write to address (num1) contents (num2) length is (num3) bytes.

only 1, 2, 4 bytes are permitted.

z or Z means: (zap, zero)

fill memory area beginning at (num1) with byte value (num2) lenght

(num3) bytes.

c or C means: (copy)

copy memory area from (num1) to (num2) with length (num3) bytes.

Empty lines and lines with a leading '#' are ignored.

If you enabled displaying dots during download, for every macro command, a

dot is displayed as well.

There is another macro file (suffix .bdmme), which is executed _after_

download of the program and before starting it. It could be used to

reverse some modifications done for preparing the download. This macro

will be executed only if there was a macro before download.

Target manipulation:

You can reset the target any time with `bdm_reset'. This is a hardware

reset, as the reset line is pulled low by the bdm interface box. This reset

is performed before any download. So you can be sure, the processor is in

a known state.

If you want to switch off bdm mode, do a `bdm_release'. This is done

automatically when quitting gdb. Code is started at that PC, where the reset

vector points to. If you did not modify vbr, this should be the

monitor program (vbr=0, vector-nr 0 -> reset startup code). The stack pointer

is set after the same mechanism.

Misc. commands (rarely needed):

`bdm_delay' sets a delay used in the driver to wait a certain amount

of time when transmitting data. It must be specified manually according

to processor speed, cache design, parallel port driving capability, etc.

This is the same as with Motorola's bd32 package.

I have found 10 to 15 be sufficient on a 486DX33 Highscreen notebook.

If the delay is too small, you will get errors. See the last chapter

"Watch-Outs" for more details on this problem.

The target module initializes the delay to 75 which is sufficient for

a i586-90. Choose a smaller value in your .gdbinit file.  `bdm_delay'

without parameter displays the current delay setting.

`bdm_init' initializes the bdm public domain interface for the parallel

port. Hopefully you will never need it.

`bdm_status' gets the status of the bdm public domain interface. If there

isn't one connected, you get no message at all. Consider this as a bug. ;-)

`bdm_log on' switches logging on. This feature has been needed for debugging.

If you ever want to know, what goes over the bdm lines when issuing a gdb

command, switch logging on and see the file `bdm-dbg.log' in the current

directory. Shutting off debugging is done with `bdm_log off'. The current

setting can be queried with `bdm_log'. The default is off.

The debug messages of the bdm driver can be enabled with

`bdm_debug_driver <lev>', where <lev> specifies, which debug level for the

bdm driver is wanted. 0 means no debug at all, 1 means 1 message for every

command, and 2 means log everything. For 2, watch your /var/adm/messages-file

before its getting too large. You might even hear your disk working during

download.... Default value is 0.

--------------------------------------------------------------------------------

C. Installation:

================

prerequisites:

- Linux version 1.1.72 or higher (versions below may work as well, but I

never tried). I am currently using kernel 1.2.4.

- module utilities. (I am using 1.1.87)

- a free parallel port. It doesn't matter, if a parallel printer is

configured in the kernel, however, you might get trouble using the printer

without reboot after changing the bdm interface and the printer cable.

The port must be jumpered on the standard addresses for

0x378 /* lpt0 or bdm0 */

0x278 /* lpt1 or bdm1 */

0x3bc /* lpt2 or bdm2 */

No interrupt assignments must be made, as this is a polling driver.

You have to do the following steps:

1. make and install the driver bdm.o

The driver sources are in gdb-bdm-patches/bdm-driver/bdm.[ch]. A

Makefile is supplied as well.

Just say make.

You get a bdm.o.

Install this as root with

# /sbin/insmod bdm.o

Check with

# /sbin/lsmod

If you dont have insmod, lsmod, rmmod, get the latest modutils.

2. make device entries in /dev

I use major number 30 for the bdm interface. This is for

iBCS-2 compatibility devices according to the latest device list,

and if this gives you a conflict, choose a free major number for the bdm

character device. Change BDM_MAJOR_NUMBER in bdm.h, and recompile the

driver.

make device entries in /dev. As root do

# mknod -m 666 /dev/bdm0 c 30 0

# mknod -m 666 /dev/bdm1 c 30 1

# mknod -m 666 /dev/bdm2 c 30 2

3. build your own cross-gdb

get gdb-4.13.tar.gz from your next GNU supplier.

$ cd <your_working_directory>

unpack it with:

$ gunzip -dc ..../gdb-4.13.tar.gz|tar xvf -

and apply the patches with

$ patch -p <gdb-bdm-patches

Configure gdb by calling configure with target

option "--target-m68k-bdm-coff". You might want to give more options as

well. See the documentation mentioned in gdb-4.13/README.

Call make to build gdb. See below for `basename' problem.

--------------------------------------------------------------------------------

D. Caveats, Watch-Outs, Gotchas:

================================

A parallel printer may not work without reboot after usage of bdm.

Breakpoints on begin and end of loops are not set correctly when debugging

on assembly level. As well as single stepping on loop boundaries. The

reason is unclear to me.

The cable size must be as short as possible. We had heavy problems with a

BDM cable (8pins, 1.5meter) when doing single stepping. They vanished suddenly

when we used a cable of 40cm. The cable between parallel port and bdm

interface is critical as well. Right now we use a box designed for dongles

to home the bdm-interface, so there is effectively no cable between port and

interface. Our experience is, that problems with the cable length will not

show up on ordinary data transmission, but on single stepping, where timing

seems to be more critical. Single stepping is used for continue after

breakpoint, next and step. So if the download works, but single stepping

and breakpoint continue not, check cable length. Thanks to Toni Zollikofer

and Josef Fuchs from Motorola Support Center in Munich for their assistence

on isolating this problem.

Increasing the delay count did not help in this case.

Concerning the delay count, there might be a weak correlation with the

kernel variable `loops_per_second', from which the famous Linus-Bogomips

are calculated, and which many drivers use for calculating software delays.

I have not tried this.

The bdm cross-gdb works for shure only for coff files. (One more reason to

change to coff :-)) Aout might work too, it depends on wether the bfd-library

gets everything out of aout as well. Positive or negative reports are welcome.

You will have to modify the configuration files for aout + bdm.

Attach and detach is not supported at the current moment. If you use it,

you might not be able to interrupt your running target application any more.

I did not disable these commands, because I dont know how. So dont use it.

Calling a procedure with the `call'-command from the gdb prompt does not

work in the cross-gdb. GDB crashes with a segmentation fault. However,

the same happened to a native GDB for Linux. So I assume, its a problem

with gdb-4.13, evtl only under Linux.

I had to modify the routine which reads in the symbols of the coff file.

My patch fixed the problem of a crashing gdb when reading the symbols of

a coff file. Maybe someone with more knowledge about the interiors of gdb

can have a look at it.

The target module is written with byte sex in mind. That means, target

(m68k) is big endian, and host (i386) is little endian. This clearly

violates GNU coding styles. It is possible to substitute the local conversion

routines (conv_buf_to_{short,long}, conf_{short,long}_to_buf,

conf_char_to_{short,long}, conv_{short,long}_to_buf). The key lies in

the extract_...() routines in findvar.c. It will be necessary when a driver

will be available on other hw-platforms with a parallel port, e.g.

sparcstations.

Before download a hard reset will be performed by the interface by pulling the

reset line down for some time. If there is a monitor running on the target

board, it might have not enough time to come up or (in our case of 332bug

for a BCC) the monitor is stopped in the middle of showing the prompt. I

had never any problems with this, except it looks strange, but if you have,

increase the value in #define TIME_TO_COME_UP in remote-bdm.c. They are

microseconds to wait after a reset.

I discovered compiling problems (conflicting definitions for basename in

gdb/defs.h and /usr/include/stdlib.h) with the latest Slackware Distribution

(2.2.) This has nothing to do with the bdm-gdb patches, but it is a problem

with Slackware.

Error messages could be improved. Sometimes, error messages with `unknown

error xxx' show up. If xxx is in the -6xx range, these messages are from

bdm interface. Check cabling to the target, power of the target, etc. If

you want to know what each code means, check bdm.h. (Yes I know that is

ugly). Sometimes you might even get no error message if there should be

one, e.g. issueing bdm_reset with target not being powered up. However

this has never been a problem in practice for me.

--------------------------------------------------------------------------------

Besides these warnings, gdb and bdm work very well together. It is much easier

to find errors when working with a debugger, than debugging with printfs, which

is often done in the embedded world.

Even if you dont want to debug, the download facility is worth using the

cross-debugger. We get download rates of more than 5Kbytes/sec. With Srecords

and 38400 kbit you will get under optimal conditions less than 2Kbyte/sec.